# Deep Research on Related Work for Multi-Roaster Coffee Batch Scheduling with Silo and Shared-Pipeline Constraints

## Executive summary

Your thesis (as specified in the uploaded problem description and formulation) is a hybrid of (i) parallel-machine, multi-product batch scheduling with sequence-dependent setups, (ii) dynamic raw-material storage (single-SKU silos with replenishment/dumping decisions), (iii) a hard shared transfer bottleneck (one GC pipeline per line with mutually exclusive operations), and (iv) inventory-driven make-to-stock production that must satisfy deterministic downstream consumption events.

Across recent OR / process-systems and pipeline-scheduling literature, I found six highly reusable “near-neighbor” works (2021–2023) that collectively cover most hard parts of your model: time discretization with inventories/backlogs, solver-scalability via staged decomposition and warm starts, bound tightening, and “bin-packing style” modeling of shared pipeline occupancy and variable-duration exclusion constraints. [1]

There is no single recent open paper that matches all of your constraints (roasters + GC silos + per-line shared pipeline + PSC 10-minute consumption + cross-line roaster). The closest matches come from chemical batch-plant scheduling and multiproduct pipeline scheduling, where “tanks/silos + transfer lines + sequence constraints + inventory feasibility” are standard. [2]

The most directly reusable components for your thesis implementation are:

- A two-step solving strategy (“first eliminate backlog/shortage, then optimize changeovers”) with warm-start transfer, which maps well to your penalty structure (shortage/tardiness dominance vs changeover/dump economy). [3]

- A staged decomposition + warm-start workflow designed for two parallel lines, which aligns with your plant layout (two lines, plus one cross-line roaster). [4]

- A discretized “bin-packing / pseudo-box packing” representation for a pipeline resource with variable-duration overlap prevention—highly relevant to your GC pipeline mutual exclusion constraint (consume/replenish/dump). [5]

- Bound-tightening via analytically derived lower bounds and explicit bounding schemes that measurably reduce MILP solve time, which is valuable given your dense time indexing and many binaries. [6]

- A hybrid “metaheuristic for sequencing + LP for quantities/inventory” pattern for scalability when exact MILP stalls, which fits your mix of “hard feasibility + soft penalties.” [7]

## Thesis problem abstraction and key constraints

The system is an 8-hour shift scheduling problem at Nestlé Trị An[8], discretized into 480 one-minute time slots. The plant has two lines, each with its own GC and RC silos; the main coupling between lines is Roaster 3, which consumes GC from Line 2 but may send RC output to either Line 1 or Line 2.

Formally (in OR terms), your model combines:

- Parallel-machine batch scheduling with sequence-dependent setup times: each roaster runs at most one batch at a time; if consecutive batches differ in SKU, a 5-minute setup is required. Processing times are SKU-dependent (13–16 minutes).

- Dynamic silo assignment and inventory feasibility for raw materials (GC): each line has 8 GC silos (capacity 3000 kg), each silo contains at most one SKU, forbids mixing, and can only change SKU after empty. The formulation explicitly models time-indexed GC inventory levels and SKU assignment binaries.

- A per-line shared GC pipeline resource with variable-duration operations: at each time slot, a line’s pipeline can do at most one of (consume, replenish, dump). Consume and replenish are fixed 2-minute operations; dump has duration 5+⌈kg/100⌉ minutes. Your formulation includes explicit pipeline occupation binary variables pl,tcon,pl,trep,pl,tdump.

- Make-to-order (MTO) + make-to-stock (MTS) coupling via roasted-coffee availability: NDG/Busta orders are due by minute 240 (end of hour 4). PSC demand is deterministic and consumed every 10 minutes (48 events/shift in the formulation).

- A penalty-driven objective combining several cost components: PSC shortage, roaster idle under safety-stock, dump, replenish, and lateness (tardiness) penalties. The formulation expresses this as Z=Zshort+Zidle+Zdump+Zrep+Ztard.

A key computational implication is that your baseline MILP is heavily time-indexed (e.g., xb,r,t for batch start decisions) and includes disjunctive “no overlap” constraints and setup constraints, which typically scale poorly without decomposition/bounds/warm-start schemes.

## Selection approach and paper set

I prioritized (a) primary, recent (2021–2023) formulations with explicit inventories and/or shared transfer resources, (b) methods that address scalability (decomposition, warm starts, matheuristics, bounds), and (c) availability of open PDFs and/or public artifacts (models/instances). The six selected works are:

- Time-bucket MILP for multistage batch plants with buffer tanks and backlog handling (2023). [9]

- MILP performance improvements for short-term batch scheduling via stepwise decomposition + warm starts on two parallel lines (2022). [4]

- Discrete-time-window MILP for a grain facility with storage and cleaning times, plus bound-tightening schemes (2022). [10]

- Discretized MILP for multiproduct pipeline scheduling, modeling overlap prevention with bin-packing-style constraints and a multi-component objective (2023). [11]

- Two-stage lot-sizing and scheduling (food/beverage) with an exact MILP baseline and a VNS/LP hybrid for scalability, plus published test instances (2023). [12]

- Rolling-horizon decomposition combining heuristics and discrete-time MILP for multi-source straight pipelines with inventory and maintenance constraints (2021). [13]

## Paper analyses

Paper A — Wallrath et al., “A time-bucket MILP formulation for optimal lot-sizing and scheduling of real-world chemical batch plants” (2023) (High similarity on inventory + changeovers + multistage; Medium on shared transfer resource)

Core problem. Roderick Wallrath[14] and coauthors propose a time-bucket MILP for simultaneous lot-sizing and scheduling in multistage chemical batch processes, intended to capture real constraints while remaining solvable at industrial scale. [15] Their core idea is to partition time into fixed-length macroperiods and flexible microperiods, aiming to combine benefits of continuous- and discrete-time scheduling representations. [16]

Formal formulation. The model includes macroperiod inventory balances for intermediates and finished products, with backlog variables to relax demand satisfaction (a useful analogue to your PSC shortage mechanism). [17] It also uses microperiod-level allocation and changeover variables (binary ypp′ls indicating product switches) and explicitly models setup phases and microperiod start times. [18]

A critical solving design is their 2-step monolithic approach with warm start:
(1) minimize total backlog; then (2) warm-start using the step-1 allocations and minimize total changeover time subject to an upper bound on total backlog (often enforcing no backlog). [19]

Methods/algorithms. Exact MILP solving with a proposed 2-step warm-start procedure; they also compare against discrete-event simulation (DES) schedules, and discuss the cost of proving optimality due to weak lower bounds (a relevant warning for your time-indexed formulation). [19]

Datasets & evaluation. They test scalability on random instances and present an industrial case study: an agrochemical formulation-and-filling process with 7 formulation lines, intermediate buffer tanks, and 7 filling lines; one-month horizon with 50 intermediates and 83 finished products. [15]

Key results. In the case study, they report that comparing an MILP solution to a DES solution indicates 17% of production capacity can be freed and on-time delivery improves. [20] They also report capacity freed at stage level when minimizing changeovers in step 2 (e.g., formulation/filling capacity freed). [21]

Limitations. Their approach still faces weak lower bounds and can have large optimality gaps; the “near-optimal quickly, proof costly” behavior is explicitly noted. [22] Also, their model is industrial-multistage but not focused on a hard shared transfer bottleneck like your per-line GC pipeline.

What you can reuse and how. This paper provides the most thesis-aligned blueprint for structuring your optimization as feasibility-first, efficiency-second:

- Adapt the 2-step warm-start idea to your penalties. Your objective includes very large penalties (PSC shortage, large tardiness) and also “economic” penalties (dump/replenish). This naturally suggests: 1) Step 1: minimize PSC shortage events + tardiness (treat dump/replenish/changeovers as secondary or ignore).
2) Step 2: fix/upper-bound shortage+tardiness at the best found level, then minimize dump/replenish + total setup/idle.
This mirrors their backlog→changeover sequence. [3]
Pitfall: step separation can produce “strange” step-1 schedules that are hard to polish if step-2 objective introduces new tradeoffs; you may need step-2 slack (bounded degradation) instead of hard equality.

- Borrow inventory-balance patterns for RC and “shortage as backlog.” Their macroperiod inventory/backlog balance equations provide a well-tested template for translating discrete consumption into (inventory, backlog/shortage) accounting. [17] In your formulation, shortage is tracked at PSC consumption events and penalized.

- Consider time-bucket in a targeted way. Your current 1-minute discretization yields many binaries. A direct time-bucket rewrite is nontrivial because you have 2-minute operations and variable dump durations. A practical compromise is to keep minute resolution only for pipeline and consumption events, while aggregating roaster decision times into coarser “eligible start times” (e.g., multiples of 2 minutes) if operationally acceptable. The pitfall is feasibility loss if processing times/setup times are not aligned with your bucket.

Links (paper + artifacts)

Paper (open PDF): https://ris.utwente.nl/ws/portalfiles/portal/322903352/1-s2.0-S0098135423002119-main.pdf
DOI: https://doi.org/10.1016/j.compchemeng.2023.108341
Code / model artifacts (LP files for step 1 & step 2): https://github.com/RoderickWR/time_bucket_MILP_formulation_for_optimal_lot_sizing_and_scheduling

Paper B — Kunath et al., “MILP performance improvement strategies for short-term batch production scheduling: a chemical industry use case” (2022) (High similarity on 2 parallel lines + staged solving + maintenance windows; Medium on storage constraints; Lower on silo/pipeline specifics)

Core problem. Sascha Kunath[23] and coauthors build a MILP scheduling model for a real-world multiproduct batch plant, then extend it into a “digital twin” including maintenance periods, campaign planning, and some storage constraints. [24] Their stated motivation is weak computational performance of monolithic MILPs.

Formal formulation. The paper is event-based (rather than your minute-indexed start variables) and includes constructs for forbidden sequences, due/earliest-start dates, and maintenance “shutdown” intervals that block equipment during windows. [25] Importantly, the plant has two nearly identical parallel production lines, which they exploit algorithmically. [26]

Methods/algorithms. Their primary contribution is an overall non-iterative multi-step decomposition approach: start with reduced scope, then incrementally add decisions across multiple MILP stages, using warm-start information from earlier stages; they also explicitly discuss tuning solver parameters (Gurobi) and warm-starting with partially specified solutions. [27]

They further propose (i) separating parallel lines and solving “single-line” products first, then (ii) inserting “multi-line” products via heuristic balancing, then (iii) final global re-optimization. [4]

Datasets & evaluation. They generate test inputs that vary the total number of batches and the counts of special constraints (multi-line products, due dates, earliest dates, campaigns, shutdowns). [26] Performance is reported in terms of feasibility, runtime budgets (targeting an overall 1-hour solving process), and MILP gap behavior under decomposition. [28]

Key results. They report that solving production lines separately yields significant performance improvement; in their experiments, decomposed MILPs can be fully solved to zero gap, largely because degrees of freedom for switching multi-line products are eliminated. [26]

Limitations. They explicitly caution that scalability to more complex layouts or more interdependent parallel routes must be analyzed carefully. [26] Also, their “storage constraints” are limited relative to your full silo + shared pipeline logic. [29]

What you can reuse and how. This is the most directly actionable paper for solver engineering of your thesis model:

- Adopt their stepwise solving structure on your two lines. Your plant is explicitly two lines with one cross-line roaster. A close mapping is:

- Step 1: Solve a reduced problem for “critical” batches—MTO NDG/Busta (due by 240) and time-constrained PSC feasibility (prevent shortage).

- Step 2: Optimize each line independently ignoring cross-line RC flow, to get good line-local schedules and pipeline usage templates.

- Step 3: Reintroduce Roaster 3’s cross-line output direction variable yb and re-optimize (or heuristic-balance) to relieve Line 1 shortages using Line 2 GC.

- Step 4: Final combined optimization with warm start from steps 1–3.

- Warm-start design for your variables. They note that providing key binaries (especially sequencing-related) can be enough for the solver to construct a valid warm start quickly. [30] In your MILP, the analogous “key binaries” are:

- xb,r,t (batch start placements)

- pipeline operation starts urep,udump and derived occupation pcon,prep,pdump

- Roaster 3 direction yb
Pitfall: A warm start that ignores some coupling constraints (e.g., GC silo-feasibility) can still be rejected; you may need a “repair” phase or a feasibility pump-like step.

- Use explicit time budgets per stage. Their staged procedure is explicitly built around hard time caps per stage (to finish within one hour). [31] This aligns with practical APS systems where schedules must be produced quickly enough to be executed.

Links (paper)

Open-access PDF mirror (German National Library page serving the paper PDF): https://d-nb.info/1258704900/34
DOI (Springer): https://doi.org/10.1007/s42452-022-04969-2

Paper C — Bayá et al., “Production Optimization in a Grain Facility through Mixed-Integer Linear Programming” (2022) (High similarity on storage + cleaning + discrete time windows; Medium on two-line production; Lower on shared-transfer bottleneck)

Core problem. Gabriel Bayá[32] and coauthors propose a discrete-time-window MILP for multi-product, multi-line production scheduling in an agri/food setting, including realistic constraints like limited storage and shelf-life, and line cleaning times. [33] They also prove NP-completeness and focus strongly on computational acceleration.

Formal formulation. The model is time-window based (weeks in their case study), includes capacity and storage constraints, and supports cleaning times between products (analogous to your fixed 5-minute setup time). [33] They derive analytical lower bounds (e.g., on total working hours, on number of shifts) and use these bounds to tighten the MILP search space. [34]

Methods/algorithms. Beyond baseline MILP, the key method is an embedded bounds-based scheme: precompute lower bounds and embed them as constraints / variable bounds to speed convergence. [6]

Datasets & evaluation. They test on 16 configurations of a real case study from a major Uruguayan grain facility (soybean harvest 2020),—17 products over 16 weeks, two production lines—where storage and cleaning constraints are integral. [33] They also report detailed instance parameter variations (mix, storage capacity, cleaning assumptions) to stress-test the model. [6]

Key results. They report average ~10% production-cost reduction vs the company’s ad hoc heuristic planning, and average ~22% elapsed-time reduction from the embedded bounds scheme (with best-case time reductions up to 99% for some instances). [35]

Limitations. Their discretization is at relatively coarse time windows (weeks), and the bottleneck structure differs from your minute-level shared pipeline; direct transfer requires careful downscaling to avoid exploding time indices.

What you can reuse and how. Their paper is the most useful template for deriving bounds that materially speed up your model:

- Bound tightening for “how many actions can occur.” Your formulation already introduces bounded helper variables (e.g., ndump∈0,30) for dump-duration linearization. The Bayá approach suggests computing additional tight bounds such as:

- Upper bounds on number of replenishes per line based on pipeline time budget: since each replenish occupies 2 minutes and the pipeline is exclusive, you have an immediate cap of ≤240 replenish-start slots per line per shift, but can sharpen it using required consume slots (2 minutes per batch) and minimal dump actions.

- Lower bounds on number of PSC batches required to satisfy deterministic consumption (given initial RC inventory and per-batch RC outputs), which can tighten the activation/search over ab.

- Bounds on changeovers (setup events) based on required SKU coverage in the shift.

- Scenario-based evaluation plan. They systematically vary major parameters to create 16 stress cases. [6] This is a strong evaluation protocol for your thesis: vary (i) PSC consumption rates, (ii) initial silo SKU distribution, (iii) dump/replenish costs, (iv) planned downtime, and report both objective and runtime/optimality gaps.

Links (paper)

Paper landing page: https://www.mdpi.com/2076-3417/12/16/8212
Open PDF: https://www.mdpi.com/2076-3417/12/16/8212/pdf
DOI: https://doi.org/10.3390/app12168212

Paper D — Wodecki et al., “Scheduling a Multi-Product Pipeline: A Discretized MILP Formulation” (2023, arXiv) (High similarity on shared pipeline mutual exclusion + variable durations + re-planning; Medium on inventory feasibility)

Core problem. Ales Wodecki[36] and coauthors present a discretized MILP for demand-driven multi-product pipeline scheduling, developed with the Czech pipeline operator CEPRO[37] for monthly planning. [38] The key modeling move is treating product transport scheduling as a bin-packing/pseudo-box packing problem via discretization of both time and space. [39]

Formal formulation. They define binary variables vebt indicating that batch b starts at time t on pipe segment e. [5] The core non-overlap constraints are bin-packing-like:

- capacity per “time bin”: b∈e​vebt≤1 for each edge and time, [5]

- feasibility of placement within horizon and explicit no-overlap via exclusion sets for variable-length boxes, [5]

- route constraints linking the same batch across multiple edges (movement through the network). [5]

They also define a multi-component objective J=αJN+βJD+γJP+θJC and include an explicit re-planning penalty that discourages deviations from a previous schedule (useful for operational rescheduling). [40]

Methods/algorithms. They develop a branch-and-cut algorithm in SCIP and report scaling behavior; they also show that the formulation scales well with commercial solvers (Gurobi) even without custom cut separation. [41]

Datasets & evaluation. They test on well-known synthetic path-graph networks and report scalability: up to 8 sites and 576 time units (24 days) “within 1000 seconds” in SCIP and up to 12 sites and 744 time units (31 days) with very small gap using Gurobi in 1 hour (as reported in the paper). [42]

Key results. The main “result” is modeling and scalability: operational constraints + discretization + branch-and-cut accelerations enable nontrivial horizons and network sizes. [42]

Limitations. Code artifacts were not located in the primary sources I accessed; the formulation is pipeline-network-centric rather than machine scheduling, so adapting it requires careful mapping of concepts.

What you can reuse and how. This paper gives you an unusually clean way to conceptualize and implement your GC shared pipeline constraints:

- Model GC pipeline operations as “boxes” in time with no overlap. In your thesis, each line’s pipeline must be mutually exclusive among replenish/consume/dump operations, and dump has variable duration 5+⌈q/100⌉. That is structurally the same as the pseudo-bin packing constraints (no two boxes overlap on a single machine/resource). [5]
Concretely: 1) Define an operation set Ol of all candidate pipeline operations (consume events induced by scheduled batches; optional replenish/dump events).
2) Create binary start variables for each operation and start time (your formulation already does this for dump/replenish via ul,s,tdump and ul,s,g,trep).
3) Impose an exclusion-set non-overlap constraint for each time slot—exactly like their Excle,t construction—to prevent overlap across variable durations. [5]
Pitfall: exclusion sets become large if naively built for all possible dump quantities; you may need to restrict dump quantities to a small set of allowed levels or use tighter linearization.

- Reuse the re-planning objective idea for operational robustness. Your system assumes full determinism within a shift. If you later extend to intra-shift updates, their “planning component” JP that penalizes deviation from a previous plan provides a principled rescheduling objective. [43]

Links (paper)

arXiv abstract: https://arxiv.org/abs/2312.11381
arXiv PDF: https://arxiv.org/pdf/2312.11381
arXiv DOI: https://doi.org/10.48550/arXiv.2312.11381

Paper E — Popović et al., “Solving a Production Lot-Sizing and Scheduling Problem from an Enhanced Inventory Management Perspective” (2023, MDPI Mathematics) (High similarity on lot-sizing + inventory penalties + scalability via hybrid heuristic; Medium on two-stage production; Lower on shared pipeline and silo purity)

Core problem. Dražen Popović[44] and coauthors study a two-stage, multi-machine lot-sizing and scheduling problem in the fruit juice industry (classified as P2SMM), extended with inventory management features: shelf-life, limited warehouse capacity, and outsourcing overflow to a 3PL. [45] The objective includes backordering, stock-level violations, warehouse overflow, setup costs, and unused production time. [45]

Formal formulation. They develop (1) a baseline MILP that jointly decides sequencing and lot sizes and (2) a hybrid VNS/LP approach: relax/remove integer scheduling constraints to turn the lot-sizing part into an LP (fast), while a Variable Neighborhood Search (VNS) searches over production sequences. [46]

Methods/algorithms. The hybrid method is an explicit “matheuristic”: VNS proposes a sequence; LP solves quantities/inventory under that sequence; iterate. [7] They also analyze which neighborhoods are most effective (e.g., insertion / change product neighborhoods). [47]

Datasets & evaluation. They generate multiple instance sets and report: - 50 small-scale instances (solved by MILP and VNS/LP), time limit 3600s for MILP;
- 50 large-scale instances where MILP is not tractable in “reasonable time;” VNS/LP is run for hours (4h in their experiments). [7]
They state that detailed instance parameters are provided as supplementary materials and can be downloaded. [47]

Key results. On small-scale instances, the hybrid VNS/LP finds the exact MILP optimum at least once for all but one instance; average cost is ~1.45% above MILP across small-scale instances, with much lower compute time for the heuristic portion. [7]

Limitations. Their feasibility structure is simpler than your “silo purity + shared pipeline” constraints. Also, their reported success relies partly on being able to “repair” infeasible quantity decisions via LP objective penalties. [48]

What you can reuse and how. This paper is a strong candidate if your full MILP becomes intractable:

- Hybridize sequencing vs flow/inventory feasibility. In your thesis, the hardest combinatorial decisions are (i) sequencing (to reduce setups and pipeline contention) and (ii) silo assignment/replenish/dump choices. A Popović-style approach would: 1) Treat roaster sequences (and possibly “which silo supplies which BOM component”) as neighborhood-search variables.
2) Given a candidate schedule/sequence, solve a continuous feasibility/penalty subproblem for quantities: GC draw amounts, dump quantities, RC inventory, shortage variables.
3) Penalize infeasibilities heavily rather than rejecting solutions outright, so the neighborhood search has gradient-like guidance.

- Use their validation protocol. They benchmark the matheuristic against MILP on small instances, then rely on the hybrid for large instances. [46] This is an excellent thesis evaluation pattern:

- build small “toy” instances (fewer silos, fewer time slots, fewer SKUs) where your MILP can be solved to optimality;

- use them to validate the heuristic;

- then scale to full 480-minute shifts.

- Potential pitfall: with your hard mutual exclusion and discrete-time pipeline operations, the LP relaxation may become too “fractional” unless you keep critical binaries (pipeline occupancy, batch start times) integer. A practical adaptation is VNS over integer start times/sequences plus an LP/MIP subproblem with fewer binaries.

Links (paper + data)

Paper landing page: https://www.mdpi.com/2227-7390/11/9/2099
DOI: https://doi.org/10.3390/math11092099
Supplementary test instances: https://www.mdpi.com/article/10.3390/math11092099/s1

Paper F — Meira et al., “A solution framework for the long-term scheduling and inventory management of straight pipeline systems with multiple-sources” (2021) (High similarity on rolling horizon + discrete-time MILP + inventory + maintenance + forbidden sequences; Medium on multi-source coupling analogous to cross-line roaster)

Core problem. William Hitoshi Tsunoda Meira[49] and coauthors address long-term scheduling of pumping and delivery operations in straight multiproduct pipelines connecting multiple sources to multiple destinations, with explicit inventory management. [13]

Formal formulation. The work emphasizes a new discrete-time MILP model that treats operational aspects like forbidden sequences and preventive tank/pipeline maintenance periods, while considering rigorous inventory management over the horizon. [13] (Full formulation details were not accessible in open primary sources during this run; the summary rests on the publisher-indexed abstract.)

Methods/algorithms. They propose a decomposition into two subproblems (initial source pumping allocation/sequence; delivery + intermediate source operations) and solve via a rolling-horizon execution procedure combining heuristic procedures and MILP models. [13]

Datasets & evaluation. Validated on an illustrative example and two real-world-based pipeline problems; solution quality is assessed mainly via feasibility and runtime adequacy (as described in the abstract). [13]

Key results. The approach reportedly obtains solutions in reasonable runtime while respecting operational constraints and detailed inventory management. [13]

Limitations. The paper is about pipeline networks and long-horizon planning rather than minute-level machine scheduling. Also, due to access limitations, the exact constraints and instance details cannot be fully unpacked here without the full text.

What you can reuse and how. The conceptual contribution is highly relevant:

- Rolling horizon for multi-shift or intra-shift replanning. Your current assumptions are deterministic for a single shift and exclude unplanned breakdowns. A Meira-style rolling horizon can extend your thesis from “one-shot schedule” to “operational APS,” where you re-optimize every H minutes using the current state (silos, in-progress roasts, current PSC running SKU) as new initial conditions.

- Two-subproblem decomposition mirrors your structure. They separate “primary source sequencing” from “downstream delivery + intermediate sources.” [13] In your plant, an analogous split is:

- Subproblem A: decide roaster sequences and start times (including setups and downtimes).

- Subproblem B: decide pipeline operations (replenish/dump timing) + exact GC silo sourcing + RC fill assignments to maintain PSC feasibility.

Links (metadata / abstract)

University of Lisbon research portal entry (includes abstract + DOI): 
https://researchportal.ulisboa.pt/en/publications/a-solution-framework-for-the-long-term-scheduling-and-inventory-m/
DOI: https://doi.org/10.1016/j.cor.2020.105143

## Reuse and adaptation plan for your thesis

This section translates the six papers into a concrete “integration roadmap” for your model as written in math_formulation_v2.md.

A pragmatic implementation strategy (minimizing risk) is to treat your thesis as a stack of coupled submodels and adopt a staged solution approach rather than pushing a single monolithic MILP to full optimality.

### Establish a baseline “correctness-first” formulation

Your formulation already defines the core time-indexed scheduling variables xb,r,t, activation variables ab for PSC batches, and pipeline occupation variables. Use this to construct a baseline solver that can produce feasible schedules (even if costly), then iteratively add sophistication:

- Start with roaster feasibility: no overlap + compatibility + setup + downtime.

- Add GC pipeline mutual exclusion and minimal GC inventory flow (enough to enforce that consume happens at batch start and cannot overlap with replenish/dump).

- Add RC inventory and PSC consumption events, initially with a simplified draw rule if needed (then refine).

### Add staged objectives and warm starts

Your penalty structure strongly suggests lexicographic behavior (avoid PSC shortage and tardiness first). Implement the Wallrath-style two-step approach:

- Step 1 (“service-level step”): minimize Zshort+Ztard (or minimize shortage indicators + tardiness minutes), ignoring dump/replenish costs except when needed for feasibility.

- Step 2 (“efficiency step”): constrain Zshort and Ztard to the best step-1 value (or allow a small tolerance), then minimize Zdump+Zrep+Zidle plus a changeover proxy (e.g., number of SKU transitions). [19]

Potential pitfall: If ctard is “effective-hard” (very large), step 1 may concentrate entirely on MTO, leaving PSC short; ensure step-1 objective includes both terms with weights or strict priority depending on your intended operational policy.

### Decompose by production line and reintroduce coupling

Kunath’s staged plan for two parallel lines maps cleanly to your topology. [4] A robust adaptation:

1) Solve Line 1 alone (R1–R2) with its pipeline and silos, treating R3→Line1 inflow as unavailable.
2) Solve Line 2 alone (R3–R5) likewise, initially forcing R3 output to Line 2 only.
3) Enable yb for R3 to route PSC output to Line 1 when it reduces Line 1 shortage/idle penalty.
4) Run a merged re-optimization with warm-start from (1)–(3).

Pitfall: Line-wise decomposition can miss global optima where Line 2 sacrifices its own performance to support Line 1; mitigate by allowing a controlled “portfolio” of cross-line batches in step 3.

### Implement pipeline mutual exclusion using bin-packing style constraints

Your GC pipeline is the core bottleneck (as your formulation summary itself highlights). Wodecki’s pseudo-box packing constraints suggest a clean implementation path:

- Treat each pipeline action (consume/replenish/dump) as a “job” on a unary machine (the pipeline).

- For fixed duration actions (consume=2, replenish=2), standard time-indexed no-overlap works.

- For dump, use (i) a bounded duration variable (your ndump=⌈q/100⌉) and (ii) exclusion sets/no-overlap constraints that depend on duration. [5]

- Consider adding a “replanning penalty” term if you implement rolling horizon; penalties for changing earlier-decided operations stabilize operations. [50]

### Add bound tightening and instance reduction

To reduce MILP size, copy Bayá’s “bound schemes” philosophy. [6] Concretely:

- Tighten PSC batch pool sizes beyond Nr,kmax=⌊480/prock⌋ using demand-based bounds: if remaining demand over the shift is limited, cap the number of activated PSC batches per SKU.

- Bound the maximum number of dump events per line by pipeline time budget and by “how many silo-SKU switches are necessary.” Since silos are single-SKU and reassignment only after empty, every SKU reassignment logically implies either full consumption or dump.

- Add precomputed lower bounds on required consume operations (at least one consume per scheduled batch start) and upper bounds on total pipeline operations.

### Scalability fallback: matheuristic (VNS/LP) layer

If the integrated MILP remains too slow, Popović provides a validated path:

- Use a metaheuristic (VNS / large neighborhood search) to propose roaster batch sequences and approximate start times. [51]

- For each candidate schedule, solve a reduced MILP/LP to decide replenishes/dumps/silo assignments/inventory flows, and evaluate the penalty cost.

- Validate on small instances that you can solve to optimality via MILP, then scale. [46]

Pitfall: In your domain, feasibility is brittle due to strict pipeline exclusivity and no-mixing silo rules. A pure LP subproblem may not detect feasibility correctly if it relaxes crucial binaries. A practical compromise is “VNS over roaster sequencing + MILP subproblem over pipeline/silo decisions,” i.e., keep the hard discrete bottlenecks inside the subproblem.

## Comparative matrix and workflow diagrams

### Comparative table

| Paper | Similarity to thesis | Core formulation | Main algorithmic contribution | Data / instances | Code / data availability | High-confidence reusable components |
| --- | --- | --- | --- | --- | --- | --- |
| Wallrath et al. 2023 [52] | High on lot-sizing+scheduling+inventory; Med on bottlenecks | Time-bucket MILP with macroperiod inventory + microperiod scheduling; backlog variables [53] | 2-step warm-start MILP: minimize backlog → minimize changeovers [3] | Industrial case study (agrochemicals), plus random instances [15] | Yes (LP files on GitHub), paper open-access [54] | 2-step objective/warm-start; inventory/backlog balance templates |
| Kunath et al. 2022 [31] | High on two-line staged solving; Med on maintenance/storage | Event-based MILP for batch plant; due dates, forbidden sequences, maintenance slots [25] | Multi-step decomposition + warm starts + solver tuning [27] | Generated test inputs varying batches and constraint counts [26] | No public code found | Stepwise line decomposition; warm-start strategy; time-budgeted solver workflow |
| Bayá et al. 2022 [35] | High on storage+cleaning; Med on multi-line | Discrete time windows MILP; storage + shelf-life + cleaning times [55] | Embedded bounds scheme to accelerate MILP convergence [6] | Real soybean facility: 17 products, 16 weeks, 2 lines; 16 scenarios [10] | No code found | Bound derivation/tightening protocol; scenario-based evaluation design |
| Wodecki et al. 2023 [11] | High on pipeline mutual exclusion; Med on inventory; Low on roasters | Discretized MILP; pseudo-bin packing with exclusion sets [5] | Branch-and-cut in SCIP; re-planning objective component [56] | Synthetic path-graph networks; scaling claims [38] | No code found; open on arXiv | Clean modeling pattern for variable-duration pipeline occupancy + rescheduling penalty |
| Popović et al. 2023 [57] | High on lot-sizing+scheduling+inventory; Low–Med on bottleneck specifics | MILP + VNS/LP hybrid; sequencing via VNS, quantities via LP [46] | Matheuristic scalability with published instance sets [47] | 50 small + 50 large instances across variants [46] | Data yes (supplementary instances), code not found [47] | Benchmarking protocol (MILP small / heuristic large); neighborhood design ideas |
| Meira et al. 2021 [13] | Med–High on rolling horizon + discrete-time inventory; Med on multi-source coupling | Discrete-time MILP + heuristics; forbidden sequences + maintenance + inventory [13] | Rolling-horizon decomposition into two subproblems [13] | Illustrative + 2 real-world-based problems [13] | Full text not confirmed open in this run | Rolling-horizon approach; decomposition pattern for multi-source coupling |

### Mermaid diagram for method-to-thesis component mapping

flowchart LR
  subgraph Thesis["Thesis model components"]
    A["Roaster scheduling<br/>parallel machines + setup + downtime"]
    B["GC system<br/>dynamic silos + shared pipeline<br/>consume/replenish/dump"]
    C["RC inventory + PSC consumption<br/>10-min events + shortages"]
    O["Penalty objective<br/>shortage + idle + dump + rep + tardiness"]
    A --> O
    B --> O
    C --> O
  end

  subgraph Papers["Reusable methods from selected papers"]
    W["Wallrath 2023<br/>time-bucket + 2-step warm-start MILP"]
    K["Kunath 2022<br/>multi-step decomposition + warm-start"]
    G["Bayá 2022<br/>bound tightening schemes"]
    P["Wodecki 2023<br/>bin-packing view of pipeline scheduling"]
    V["Popović 2023<br/>VNS/LP matheuristic"]
    R["Meira 2021<br/>rolling horizon decomposition"]
  end

  W --> A
  W --> C
  W --> O

  K --> A
  K --> O

  G --> A
  G --> B
  G --> C

  P --> B
  P --> O

  V --> A
  V --> C

  R --> B
  R --> O

### Consolidated source links

Thesis artifacts (uploaded):
- Thesis Problem Description.md (problem statement + costs + pipeline durations + roaster layout)
- math_formulation_v2.md (mathematical formulation, variables, objective, constraint categories)

Selected papers / artifacts:
- Wallrath et al. 2023 (time-bucket MILP, open PDF): https://ris.utwente.nl/ws/portalfiles/portal/322903352/1-s2.0-S0098135423002119-main.pdf
- Wallrath et al. supplemental LP files (GitHub): https://github.com/RoderickWR/time_bucket_MILP_formulation_for_optimal_lot_sizing_and_scheduling
- Kunath et al. 2022 (open PDF mirror): https://d-nb.info/1258704900/34
- Bayá et al. 2022 (MDPI page): https://www.mdpi.com/2076-3417/12/16/8212   | PDF: https://www.mdpi.com/2076-3417/12/16/8212/pdf
- Wodecki et al. 2023 (arXiv PDF): https://arxiv.org/pdf/2312.11381
- Popović et al. 2023 (MDPI page): https://www.mdpi.com/2227-7390/11/9/2099
- Popović et al. 2023 supplementary instances: https://www.mdpi.com/article/10.3390/math11092099/s1
- Meira et al. 2021 metadata/abstract: https://researchportal.ulisboa.pt/en/publications/a-solution-framework-for-the-long-term-scheduling-and-inventory-m/

[1] [2] [3] [9] [15] [16] [17] [18] [19] [20] [21] [22] [23] [37] [52] [53] https://ris.utwente.nl/ws/portalfiles/portal/322903352/1-s2.0-S0098135423002119-main.pdf

https://ris.utwente.nl/ws/portalfiles/portal/322903352/1-s2.0-S0098135423002119-main.pdf

[4] [24] [25] [26] [27] [28] [29] [30] [31] https://d-nb.info/1258704900/34

https://d-nb.info/1258704900/34

[5] [11] [32] [38] [39] [40] [41] [42] [43] [50] [56] https://arxiv.org/pdf/2312.11381

https://arxiv.org/pdf/2312.11381

[6] [10] [33] [34] [44] [49] [55] https://www.mdpi.com/2076-3417/12/16/8212/pdf?version=1660740338

https://www.mdpi.com/2076-3417/12/16/8212/pdf?version=1660740338

[7] [8] [12] [45] [46] [47] [48] [51] [57] https://www.mdpi.com/2227-7390/11/9/2099

https://www.mdpi.com/2227-7390/11/9/2099

[13] [14] [36] https://researchportal.ulisboa.pt/en/publications/a-solution-framework-for-the-long-term-scheduling-and-inventory-m/

https://researchportal.ulisboa.pt/en/publications/a-solution-framework-for-the-long-term-scheduling-and-inventory-m/

[35] https://www.mdpi.com/2076-3417/12/16/8212

https://www.mdpi.com/2076-3417/12/16/8212

[54] https://github.com/RoderickWR/time_bucket_MILP_formulation_for_optimal_lot_sizing_and_scheduling

https://github.com/RoderickWR/time_bucket_MILP_formulation_for_optimal_lot_sizing_and_scheduling