REVISED THESIS SCOPE MEMO

For Instructor Review and Approval

| Student | Nim Tien Dat (IELSIU22313) |
| --- | --- |
| Advisor | ThS. Tran Van Ly |
| Program | Bachelor of Engineering, School of Industrial Engineering, IU VNU-HCM |
| Date | March 2026 |

# 1. Summary of the Original Thesis Proposal

## 1.1 Title and Research Question

Original title: "Integrated Batch Roasting Scheduling with Green Coffee Silo Constraints: A Comparative Study of MILP and Constraint Programming at Nestle Tri An."

The original thesis proposed a deterministic, offline optimization model for within-shift batch roasting scheduling. The central research question was: does CP-SAT (Google OR-Tools) outperform MILP (IBM ILOG CPLEX) on this specific problem class, and why? The hypothesis was grounded in the disjunctive nature of the pipeline mutex constraint, which structurally favors CP's native NoOverlap propagation over MILP's Big-M linearization.

## 1.2 Physical Model Scope

The original formulation modeled the full physical complexity of the Nestle Tri An roasting system, including:

GC (Green Coffee) Silo System: 16 dedicated single-SKU silos (8 per line), each with 3,000 kg capacity. The model tracked: inventory levels per silo per time slot, dynamic SKU assignment (silos could be reassigned to a new SKU only after being emptied to 0 kg), single-SKU purity (no mixing allowed), and BOM sourcing rules (each batch draws GC from 1-2 silos per GC-SKU, with split-sourcing permitted only if at least one silo is emptied).

Shared GC Pipeline (3 operation types): Each line had one shared pipeline supporting three mutually exclusive operations: consume (2 min, draw GC into roaster), replenish (2 min, add 500 kg from warehouse to silo), and dump (variable duration: 5 + ceil(q/100) min, where q is dump quantity in kg). The variable-duration dump was the most complex modeling element — it required linearization via auxiliary integer variables in MILP and was expressed as optional interval variables with size functions in CP-SAT.

RC (Roasted Coffee) Silo System: 8 individual RC silos (4 per line, each 5,000 kg capacity), tracked at silo level with fill/draw rules: lowest-stock-first fill priority, highest-stock-first consumption priority (buffer silo 4 consumed first), and a look-ahead feasibility constraint preventing batch starts when all RC silos are full.

Three shift types: Normal production, mid-shift SKU switch (RC threshold < 15,000 kg before switch), and 4-hour changeover (with 2-hour PSC restart delay after changeover completion).

Units: All quantities in kg (continuous). Roasting time: 13-16 minutes per batch, varying by SKU.

## 1.3 Objective Function

The objective was to minimize total penalty cost (VND) across five weighted components: PSC shortage penalty (600,000 VND/min), roaster idle penalty (100,000 VND/min when RC below 10,000 kg safety stock), dump penalty (500,000 VND fixed + 10,000 VND/kg), replenish cost (100,000 VND per event), and MTO tardiness penalty (very large per-minute penalty calibrated to behave as near-hard constraint).

## 1.4 Solution Method

Two exact solvers were implemented in parallel: MILP via IBM ILOG CPLEX (formal mathematical reference) and CP-SAT via Google OR-Tools (proposed method). Both were compared on 21 historical shift instances from the factory's best-performing week, evaluated on three metrics: total penalty cost, solve time, and scalability.

## 1.5 Model Scale

The MILP formulation contained approximately 400,000-500,000 decision variables (dominated by time-indexed batch assignment binaries), 38 constraint groups (C1-C38), and was expected to require 30-minute solve time limits. 21 historical shift instances served as the validation dataset.

# 2. Instructor Feedback and Rationale for Change

## 2.1 Critique 1: Scope Too Wide, Not Deep

The instructor observed that the original thesis tries to model the entire roasting operation at surface level — GC silo SKU management, variable-duration dump linearization, RC silo fill/draw rules, SKU switch thresholds, changeover logic, pipeline mutex with three operation types — resulting in a formulation that is broad but shallow. For an undergraduate thesis, this risks hitting computational walls on the full-scale instances and spending the thesis explaining why the solver could not handle the problem, rather than producing meaningful results.

The instructor specifically recommended: simplify the silo constraints (treat GC as unlimited, no dump/replenish), drop the 4-hour changeover (only retain PSC↔MTO switching), and simplify the pipeline to consume-only. The physical model should be simple enough that the thesis can go deep on the solution method.

## 2.2 Critique 2: Comparing Two Exact Solvers Is Not a Methodological Contribution

The instructor pointed out that comparing MILP (CPLEX) and CP-SAT (OR-Tools) is essentially a software benchmark, not a research contribution. Both are exact solvers solving the same mathematical formulation. Showing that one runs faster on a specific instance set is an implementation decision, not a methodological insight. The committee would reasonably ask: "So what? You picked the faster tool."

## 2.3 Instructor's Constructive Direction

The instructor hinted at two specific directions:

Focus on what brings practical value: the pipeline mutex constraint (simplified to consume-only) is still non-trivial with 60% utilization on Line 2. The demand pattern (fixed PSC consumption rate, MTO deadlines) creates real scheduling tension. The model should be expressed in batches, not kg — simpler, more intuitive, and sufficient for the scheduling decisions.

Focus on real-time management under uncertainty: unplanned stoppages (UPS) are the dominant cause of throughput loss at the factory. The thesis should address how to handle UPS — not just produce a static deterministic schedule. This shifts the contribution from "what is the optimal schedule" to "how should the system react when the schedule is disrupted."

# 3. What Changed: Detailed Comparison

| Aspect | Original Proposal | Revised Scope |
| --- | --- | --- |
| Research question | Which solver is better for this problem? (CP-SAT vs. MILP) | Which reactive scheduling strategy handles disruptions best? (Dispatching vs. CP-SAT vs. DRL) |
| Contribution type | Solver benchmark (computational) | Methodology comparison (three paradigms) |
| Uncertainty | None — fully deterministic | UPS events (stochastic, exponential inter-arrival) |
| GC silo system | 16 silos, inventory tracking, SKU purity, dynamic reassignment, dump, replenish | Removed entirely — GC supply unlimited |
| Pipeline operations | 3 types: consume (2 min), replenish (2 min), dump (5 + ceil(q/100) min, variable) | Consume only (3 min, fixed duration) |
| RC tracking | Individual silo-level (kg), fill/draw rules, buffer silo logic | Aggregate batch counter per line (integer) |
| Units | Kilograms (continuous) | Batches (integer) |
| Roasting time | 13-16 min per batch (SKU-dependent) | 15 min fixed (uniform, all SKUs) |
| Shift types | Normal, SKU switch, 4-hour changeover | Normal and PSC↔MTO switch only (changeover dropped) |
| Objective | Minimize multi-component penalty (VND) | Maximize throughput − tardiness penalty |
| Solution methods | MILP (CPLEX) vs. CP-SAT (OR-Tools) | Dispatching vs. CP-SAT re-solve vs. DRL (PPO) |
| Model size | ~500,000 variables, 38 constraint groups | ~200 variables, 11 constraint groups |
| Solve time | Minutes to hours (30-min time limit) | < 1 second per re-solve |
| Validation data | 21 historical shift instances (primary) | 9,000 simulated runs (primary) + 21 instances (secondary validation) |
| Planned downtime | Batch in-progress completes before downtime | Batch must finish before downtime; no start if cannot complete |
| UPS handling | Excluded from scope (limitation L1) | Central research focus — batch cancelled, GC lost, scheduler decides restart |

# 4. What Is Preserved

The following elements carry over from the original proposal unchanged or with minor simplification:

Industrial setting: Nestle Tri An facility, Dong Nai province. Same factory, same roasting stage, same data access.

5 roasters on 2 lines: R1, R2 (Line 1); R3, R4, R5 (Line 2). Same eligibility structure: R1 (PSC, NDG), R2 (PSC, NDG, Busta), R3-R5 (PSC only).

R3 cross-line architecture: R3 always consumes from Line 2 pipeline, RC output routable to Line 1 or Line 2. Now also used as experimental factor (fixed vs. flexible routing).

Shared pipeline mutual exclusion: Simplified from 3 operation types to consume-only, but the NoOverlap constraint structure is unchanged. Pipeline remains the core physical bottleneck.

MTO/MTS demand structure: NDG/Busta as MTO with due dates; PSC as MTS with continuous consumption. NDG/Busta output still delivered directly (does not enter RC stock).

Sequence-dependent setup time: 5 minutes between any different SKU pair on the same roaster. Unchanged.

CP-SAT solver: Google OR-Tools CP-SAT remains the optimization engine. It is now used for reactive re-solving rather than as one of two competing solvers.

21 historical shift instances: Still available for deterministic baseline validation. No longer the primary experimental dataset.

8-hour shift horizon (480 time slots): Unchanged.

Only 1 PSC SKU per shift: No PSC-PSC switching within a shift.

# 5. New Thesis Scope in Detail

## 5.1 Revised Title

Reactive Batch Roasting Scheduling Under Unplanned Stoppages: A Comparative Study of CP-SAT Re-optimization and Reinforcement Learning at Nestle Tri An

## 5.2 Simplified Physical System

| Parameter | Value |
| --- | --- |
| Roasters | 5 total: R1, R2 (Line 1); R3, R4, R5 (Line 2) |
| Eligibility | R1: PSC, NDG. R2: PSC, NDG, Busta. R3-R5: PSC only. |
| Roasting time | 15 min per batch (uniform, all SKUs) |
| Pipeline | Consume only, 3 min per batch, NoOverlap per line. R3 always uses Line 2 pipeline. Consume is concurrent with roast start (pipeline busy for first 3 of 15 min). |
| Setup time | 5 min between any two different SKUs on same roaster |
| R3 output routing | Decision variable per batch: Line 1 or Line 2. Also experimental factor: fixed (always Line 2) vs. flexible. |
| GC supply | Unlimited — no silo tracking, no dump, no replenish |
| PSC demand | Fixed consumption rate per SKU (e.g., 1 batch / 5.1 min). Precomputed schedule. One PSC SKU per shift. |
| MTO demand | 3-5 batches (NDG/Busta), soft due date at slot 240. Output delivered directly, does NOT enter RC stock. |
| RC inventory | Aggregate batch counter per line (integer). Stockout = hard in deterministic, soft in reactive. Overflow = always hard. |
| Objective | Maximize PSC throughput − tardiness penalty |
| Planned downtime | Known input. Batch must finish before downtime starts. No mid-batch pause. |

Core bottleneck: Line 2 pipeline utilization = 3 roasters × 3 min / 15 min cycle = 60%. Only 6 minutes of slack per cycle. Any UPS causes cascading pipeline delays across R3, R4, R5.

## 5.3 Unplanned Stoppages (UPS) — The Stochastic Element

UPS events are the central uncertainty in the revised thesis. When a UPS occurs on a roaster mid-batch:

1. The batch is cancelled entirely. No pause/resume. The green coffee already consumed is lost (sunk cost). The batch must be restarted from scratch if desired (new 3-min consume + full 15-min roast).

2. The roaster goes DOWN for a random duration drawn from a parameterized distribution.

3. RC output is NOT credited — the incomplete batch produces nothing. RC stock continues to deplete via consumption events while the roaster is down.

4. When the roaster recovers, the scheduler decides whether to restart the cancelled batch or schedule a different batch. This is a decision variable, not automatic. If the cancelled batch was MTO (NDG/Busta), skipping it increases tardiness penalty.

UPS parameters are synthetic (exponential inter-arrival with rate λ, parameterized duration with mean μ), based on literature MTBF/MTTR estimates. Not calibrated to plant-specific data — this is an acknowledged limitation. Sensitivity analysis across the (λ, μ) grid compensates.

## 5.4 Research Question

For reactive batch scheduling under unplanned stoppages with a shared pipeline constraint, how do optimization-based (CP-SAT) and learning-based (DRL) reactive strategies compare against each other and against rule-based dispatching, across varying disruption intensities?

## 5.5 Three Reactive Scheduling Strategies

| Strategy | Type | Description |
| --- | --- | --- |
| Dispatching | Rule-based | Priority rules: EDD for MTO, then fill-PSC for RC stock. Myopic — no lookahead, no learning. Represents current operator practice. |
| CP-SAT Re-solve | Optimization | Event-triggered: when UPS occurs, freeze completed batches, re-solve remaining horizon with CP-SAT. Optimal for each instance but stateless — no memory of past disruptions. |
| DRL (PPO) | Learning | MaskablePPO trained on thousands of simulated UPS scenarios. Action masking ensures feasibility. Instant inference. Stateful — can learn distributional patterns in disruptions. |

Why CP-SAT vs. DRL is a meaningful comparison: CP-SAT re-solve is stateless — each re-plan is solved from scratch with no memory of prior disruptions. DRL is stateful — the trained policy implicitly encodes learned patterns (e.g., "after 2 UPS in the first half, Line 2 stock will be low — route R3 to Line 2 preemptively"). The thesis investigates whether DRL's learned patterns outweigh CP-SAT's per-instance optimality under high disruption rates.

## 5.6 Experimental Design

| Factor | Levels | Rationale |
| --- | --- | --- |
| UPS rate λ | 0, 1, 2, 3, 5 events/shift | From no disruption to heavy disruption |
| UPS duration μ | 10, 20, 30 min (mean) | From quick repair to significant downtime |
| Strategy | Dispatching / CP-SAT / DRL | Three fundamentally different reactive paradigms |
| R3 routing | Fixed (L2) / Flexible | Measures value of cross-line flexibility under disruption |

Full factorial: 5 × 3 × 3 × 2 = 90 cells × 100 replications = 9,000 simulation runs. Same 100 UPS realizations applied to all 3 strategies per cell (paired comparison — differences are purely from strategy, not randomness).

KPIs: total throughput (PSC batches), stockout count and duration, MTO tardiness, computation time per decision.

# 6. Implementation Plan (10 Weeks)

| Weeks | Task | Deliverable |
| --- | --- | --- |
| 1-2 | Simulation environment + deterministic CP-SAT model + dispatching heuristic | Full-shift schedule (no UPS) |
| 3-4 | UPS simulation engine + CP-SAT reactive re-solve loop + experiments (dispatching vs. CP-SAT) | Minimum viable thesis |
| 5-7 | Gymnasium environment + MaskablePPO training + action masking + DRL evaluation | Trained DRL agent |
| 8-9 | Full factorial experiments (9,000 runs) + statistical analysis + visualizations | Results chapter |
| 10 | Thesis writing finalization | Complete thesis |

Risk mitigation: CP-SAT reactive (end of week 4) constitutes a defensible thesis on its own (dispatching vs. CP-SAT comparison). DRL (weeks 5-7) elevates the contribution but is not required for a passing thesis. If DRL training encounters difficulties, the thesis proceeds with two strategies. Additionally, ALNS (Adaptive Large Neighborhood Search) is documented as potential future work for expandability — it is the most natural metaheuristic for schedule repair and could serve as a fourth strategy in a follow-up study.

# 7. Request for Approval

I request approval to proceed with this revised scope. Specifically, I seek confirmation on the following points:

(1) Simplified physical model: Is treating GC as unlimited, tracking RC as aggregate batch counter, and using consume-only pipeline (3 min) appropriate and sufficient for the scheduling research question?

(2) Three-strategy comparison: Does the comparison of dispatching (baseline) vs. CP-SAT re-optimization vs. DRL (PPO with action masking) constitute a valid methodological contribution?

(3) Simulated UPS events: Are synthetic UPS parameters (literature-based MTBF/MTTR) acceptable given the unavailability of plant-specific failure data? The sensitivity analysis across 5 UPS rates × 3 durations is designed to compensate.

(4) R3 routing as experimental factor: Does comparing fixed vs. flexible R3 routing add analytical value without overcomplicating the study?

(5) ALNS as future work: Is documenting ALNS as a potential fourth strategy (expandability for future research) acceptable, with implementation only if time permits?

Student: ___________________________

Nim Tien Dat

Advisor: ___________________________

ThS. Tran Van Ly